Great. Let's build Week 1: 7 September 2026 as the model lesson that we can then replicate for Weeks 2–7.

Pearson's guidance explicitly supports introducing safe electronic working, emergency procedures, schematic standards, prototyping, simulation and physical testing within Learning Aim A. (Pearson Qualifications)

Week 1 — Unit 19 Launch

“Can You Make It Work?”

Workplace story

Students are told:

You have joined the electronics engineering team at NovaTech Systems as junior electronics technicians.

A prototype control board has failed its bench test. The senior engineer is unavailable, so your team has been asked to inspect the circuit, identify the fault, repair it and provide evidence that the board is safe and working.

This becomes the hook for the whole lesson.

The students are not told the fault.

Their first engineering instruction is:

“Do not power the circuit until you are satisfied it is safe to do so.”

That immediately creates a reason to learn safety, schematic interpretation, pre-power checks and measurement.

1. Lesson at a glance

| Item | Detail |
| --- | --- |
| Date | Monday 7 September 2026 |
| Duration | 3 hours including 20-minute break |
| Teaching time | 160 minutes |
| Learning Aim | A – Explore the safe operation and applications of analogue devices and circuits |
| Focus | A1 foundation + introduction to A5/A6 |
| Workplace role | Junior electronics technician |
| Core challenge | Can You Make It Work? |
| Pedagogy | Problem-based, exploratory, practical, collaborative |
| Portfolio output | PoE 01 – Prototype Fault Investigation |
| Assignment relationship | Introduce the final assessment journey and evidence expectations |
| Assessment | Formative; baseline practical observation + PoE |

2. Learning objectives

By the end of the session, learners should be able to:

Know

- What Learning Aim A is about.

- Why safe working procedures are essential when constructing/testing circuits.

- The purpose of isolation, emergency procedures and reporting.

- Basic electronic schematic conventions.

- The basic function of a breadboard.

- The purpose of a multimeter.

Demonstrate

Learners should be able to:

- Identify several common components.

- Read a simple circuit schematic.

- Perform a pre-power inspection.

- Build/check a simple low-voltage circuit.

- Use a multimeter for continuity and voltage checks.

- Apply a systematic troubleshooting sequence.

- Safely identify, repair and verify a simple circuit fault.

- Record their engineering work professionally.

3. The central practical challenge

“Can You Make It Work?”

I would deliberately make the circuit simple enough to understand but difficult enough to troubleshoot.

Suggested circuit

A simple:

5 V supply → switch → resistor → LED → 0 V

with perhaps an additional test point.

For different groups, introduce one fault:

Fault A

LED reversed.

Fault B

Resistor wrong value / open circuit.

Fault C

Jumper wire misplaced.

Fault D

Broken/poor connection.

Fault E

Supply connection missing.

Fault F

Incorrect breadboard row.

The important point is that the fault must be discoverable through systematic engineering practice, rather than obscure enough to become a guessing game.

4. The 160-minute lesson structure

Phase 1 — Hook and diagnostic

0–15 min

Display a photograph of a prototype PCB/breadboard with a red FAILED TEST label.

Ask:

“You are told this circuit doesn't work. What is the first thing you do?”

Don't immediately accept answers.

Students individually write their first three actions.

Then compare answers with a partner.

Key teaching point

An engineer doesn't immediately start pulling components out.

The process should begin with:

Safety → Observation → Information → Prediction → Test

Phase 2 — Safety scenario

15–35 min

Give students the scenario:

“Smoke is seen coming from a circuit on the bench. What should you do?”

Give groups cards containing actions such as:

- Continue troubleshooting

- Disconnect/isolate supply

- Inform tutor/responsible person

- Raise alarm where necessary

- Touch the circuit to see if it is hot

- Evacuate if instructed

- Record/report the incident

- Restart the power

Students must arrange them in the correct order.

Then demonstrate the actual laboratory arrangements:

- Main/bench power isolation

- Emergency stop where present

- Heat-source isolation where relevant

- Fire alarm

- Assembly point

- First-aid arrangements

- Reporting route

Important

This should be specific to your actual laboratory, not generic.

Phase 3 — Schematic → physical circuit

35–55 min

Show the challenge schematic.

Ask:

“Before we touch the breadboard, what can we predict?”

Students identify:

- Supply

- Switch

- Resistor

- LED

- Ground/0 V

- Current path

Then introduce the idea:

A schematic is the engineer's instruction/communication language.

Quick activity:

“Point to it”

Tutor points to a schematic symbol.

Students must identify the corresponding physical component.

Then reverse it:

Tutor holds up a physical component.

Students identify its schematic symbol.

Phase 4 — Breadboard mini-challenge

55–75 min

Demonstrate only what they need:

- Breadboard power rails

- Connected rows

- Centre channel

- Component placement

- LED polarity

- Safe jumper-wire use

Don't turn this into a 40-minute breadboard lecture.

Student challenge

Build the circuit WITHOUT connecting the power supply.

They then call the tutor over for a pre-power inspection.

This is important.

The student should not be rewarded for simply making the LED light.

They need to establish:

“I know it is safe to power.”

20-minute break

75–95 min

Phase 5 — Power, measure and troubleshoot

95–130 min

Now the engineering investigation starts.

Students:

- Verify PSU setting.

- Connect supply.

- Observe behaviour.

- Record symptom.

- Predict likely fault.

- Take measurements.

- Locate fault.

- Correct it.

- Retest.

Introduce the troubleshooting sequence:

OBSERVE → PREDICT → TEST → LOCATE → FIX → VERIFY

Tutor questioning

Instead of:

“What's wrong?”

ask:

“What evidence makes you think that?”

Then:

“What measurement could prove/disprove your theory?”

This begins developing the analytical behaviour we eventually need for higher grades.

Phase 6 — Make it harder

130–145 min

Once a pair fixes its first fault:

Challenge 2

Introduce a second fault or swap their circuit with another group's.

Now they need to repeat the process independently.

Optional competitive element

Put a simple leaderboard on the board:

| Team | Fault identified | Evidence-based? | Safe process? |
| --- | --- | --- | --- |
| Team 1 | ✅ | ✅ | ✅ |
| Team 2 | ✅ | ❌ | ✅ |
| Team 3 | ✅ | ✅ | ✅ |

But don't make fastest = best.

Award points for:

Safe + systematic + evidence-based

rather than simply speed.

Phase 7 — Build the Portfolio of Evidence

145–155 min

Students create:

PoE 01 — Prototype Fault Investigation

They capture:

1. Circuit schematic

Annotated with important points.

2. Photograph

Working/final circuit.

3. Measurements

Example:

| Test point | Expected | Measured |
| --- | --- | --- |
| Supply | 5.0 V | 4.98 V |
| LED resistor | — | — |
| LED voltage | — | — |

4. Fault diagnosis

What did you think was wrong?

5. Evidence

What measurement/check proved it?

6. Repair

What did you change?

7. Verification

How did you prove it now worked?

8. Reflection

Complete:

I originally thought...
I tested...
I discovered...
Next time I would...

This is the beginning of the engineering record, not a formal graded assignment section.

Phase 8 — Exit ticket

155–160 min

Three questions:

1. What must you do before powering a circuit?

2. Why shouldn't an engineer simply start replacing components?

3. What is the difference between a guess and an engineering diagnosis?

Expected conceptual answer to #3:

A diagnosis is supported by evidence.

That last question is particularly useful because it plants the culture we need for the entire unit.

5. Differentiation

For learners needing additional support

I would prepare a "Build & Diagnose" support card with:

Before power

☐ Check supply voltage
☐ Check polarity
☐ Check component orientation
☐ Check resistor value
☐ Check jumper connections
☐ Check for unintended shorts
☐ Ask tutor to inspect

Troubleshooting

1. What should happen?

↓
2. What actually happens?

↓
3. What could cause the difference?

↓
4. What can I measure?

↓
5. What does the measurement tell me?

This supports students without simply giving them the answer.

SEND-friendly design

Use:

- Large clear diagrams

- Minimal text blocks

- Numbered actions

- Colour-coded wires

- Component photographs

- Symbol + photograph together

- Vocabulary bank

- Demonstration before independent work

- Paired roles

- Frequent tutor checkpoints

- Oral explanation as an alternative rehearsal before writing

6. Stretch and challenge

For stronger learners:

Level 1

Find one known fault.

Level 2

Find two faults.

Level 3

Find a fault without being given the circuit schematic.

Level 4

Explain why a particular measurement proves the fault.

Level 5

Design a simple diagnostic procedure that another team could follow.

That gives differentiation without requiring a completely different lesson for different students.

7. Equipment

For approximately 2 learners per workstation:

Essential

- Breadboard

- Low-voltage DC bench PSU

- LEDs

- Resistors

- Jumper wires

- Switches

- Multimeter

- Crocodile leads where appropriate

- Printed schematic

Tutor

- Faulty circuit examples

- Correct circuit

- Spare components

- Component tester/multimeter

- Demonstration breadboard

- Camera/document visualiser if available

Digital

- PCs

- Multisim

- Optional Tinkercad/Wokwi

I would not make simulation the main activity in Week 1. It is introduced briefly so students understand that the same engineering process exists in a virtual environment.

Pearson specifically expects schematic capture/simulation and physical testing within Learning Aim A, so introducing both early is worthwhile. (Pearson Qualifications)

8. Tutor materials to prepare

For this week, I would create a compact pack:

Student-facing

W1-01 – Engineering Challenge Brief
W1-02 – Safety & Emergency Scenario
W1-03 – Circuit Schematic
W1-04 – Pre-Power Checklist
W1-05 – Troubleshooting Flowchart
W1-06 – Measurement Record
W1-07 – Portfolio of Evidence 01 Template
W1-08 – Exit Ticket

Tutor-facing

W1-T01 – Detailed Lesson Plan
W1-T02 – Fault Circuit Master + Fault Keys
W1-T03 – Expected Measurements
W1-T04 – Tutor Observation Checklist
W1-T05 – Differentiation/SEND Intervention Guide
W1-T06 – Suggested Questions
W1-T07 – Equipment Setup Checklist

9. Portfolio of Evidence — Week 1 template

This is what I'd actually give students.

PoE 01 — Prototype Fault Investigation

Student:
Date:
Team:
Circuit ID:

A. The engineering problem

What were you asked to investigate?

B. Safety checks

List three checks you made before powering the circuit.

C. What did you expect?

Describe what the circuit should do.

D. What actually happened?

Describe the fault/symptom.

E. Troubleshooting

| Test/check | Result | What did it tell you? |
| --- | --- | --- |

F. Diagnosis

What was the fault?

What evidence supports your diagnosis?

G. Repair

What did you change?

H. Verification

How did you prove the circuit was working?

I. Engineering reflection

One thing I did well:
One thing I found difficult:
One thing I will do differently next time:

10. What I would NOT do in Week 1

This is just as important.

I would not spend the session:

- going through every Pearson assessment criterion

- teaching semiconductor physics

- teaching extensive breadboard theory

- teaching every multimeter function

- delivering a long health-and-safety presentation

- asking students to write a long assignment

- trying to cover diodes/transistors/op-amps

Instead:

Give them a problem they care about solving, then teach them what they need to solve it.

That establishes the culture for the remaining six weeks.

The Week 1 learning cycle

REAL PROBLEM

↓

IS IT SAFE?

↓

WHAT SHOULD HAPPEN?

↓

BUILD / CHECK

↓

POWER

↓

MEASURE

↓

FAULT

↓

FIX

↓

VERIFY

↓

DOCUMENT

That becomes the DNA of the entire Learning Aim A.

And it makes the later diode, transistor and op-amp sessions feel like progression in engineering capability, rather than three separate chunks of BTEC theory.

The next week can then build directly from this foundation: “The One-Way Street” — a workplace diode investigation, with the first genuine assignment component being developed alongside the practical.